# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Supply of  Measurement Test-bed  equipment  for water distribution system monitoring	p 1 /…

Publication reference:  CN1-S.O 2.3-SC011/3

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required |  | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- | --- |
|  | Measurement test-bed for water distribution system monitoring consisted of: |  |  |  |  |
| 1. | Main unit | - Anodized aluminium structure; - Main element in stainless steel; - Pipe network of test pipes with the following characteristics: - At least one aluminium pipe with an outer diameter equal to 16 mm; - At least one methacrylate pipe with an outer diameter equal to 16 mm; - At least one PVC pipe with an outer diameter equal to 16 mm; - At least one PVC pipe with an outer diameter equal to 20 mm; - At least one PVC pipe with an outer diameter equal to 25 mm; - At least two magnetic displacement sensors with a measuring range between 0 and 1 m; - At least two water manometers with a length equal to 1000 mm; - At least two pressure sensors, with a range between 0 and 30 PSI; - At least one flow sensor, with a range between 2 and 150 l/min; - At least one valve for distribution of flow to the network; - A mobile hydraulics bench; - One centrifugal pump with a power of at least 0.36 kW, and able to distribute between 30 and 80 l/min at 20,1 - 12.8 m; - One sump tank with a capacity of at least 160 l; - One small channel with a capacity of at least 8 l; - Must include options for Real-time control and SCADA. - Must also contain a suitable control interface unit which will provide an interface between the main test-bed unit and the data acquisition board, for all of the sensor and control/actuator signals; Must contain a simplified process diagram representation on the front panel; |  |  |  |
| 2. | Data acquisition board | - PCI Express Data board, must be possible to be placed in a computer slot; - Analog input channels: - At least 16 single-ended or 8 differential; - Resolution of at least 16 bits; - Sampling rate of up to 250 kilo samples/s; - Input range of ±10 V; - Analog output channels: - At least 2 channels; - Resolution of at least 16 bits; - Output range of ±10 V; - Digital input/output channels: - At least 24 channels; - Sample clock frequency between 0 and 100 MHz; At least 4 counters/timers, with a resolution of at least 32 bits; |  |  |  |
| 3. | Control and acquisition software | - Suitable software for computer control, data acquisition, and data management; - Must be compatible with industry standards; - Must allow management, comparison, processing, storage of data; - Must have a sampling velocity of up to 250 kilo samples/s, compatible to the data acquisition board; - Must allow calibration for all involved sensors; - Must allow visual graphic representation, control and manipulation of the test-bed and all of its process parameters; - Must allow modification of parameters; Must allow access to different work levels via passwords; |  |  |  |
| 4. | Electric cabling | Suitable for normal operation of the entire system. |  |  |  |